CLEANING & MAINTAINING YOUR POLYURETHANED TIMBER FLOOR

Polyurethaned floors are sanded, trowel filled & coated with 3 coats of moisture cured (solvent based) polyurethane of either a matt or satin finish.   In general, reapplication of polyurethane coatings is necessary only when damage occurs or after 7-10 years of typical wear. Polyurethane is not waterproof, only water-resistant and excess water will seep through the coatings causing damage. These floors are not waxed, polished or buffed to maintain them – just cleaned.

IN GENERAL

Preferably use dry (spray on & wipe off), not wet (mop & bucket), cleaning methods.

- Keep dirt and grit off the floor as it can be ground in and damage coatings.

- Use protective mats inside and outside exterior doorways.

- Wipe up spills ASAP with a slightly damp cloth and remove excess moisture with a dry mop or cloth immediately.

- Use mats and/or floor felts under heavy furniture and movable objects (chairs, etc.) to prevent markings.

- Don’t use excessive water or let water sit on timber flooring. On completion of cleaning/mopping, the floor should be dry.

- Check your vacuum cleaner wheels and head won’t leave scratches, use the brush rather than the hard head.

- Don’t use steam mops, soaps, detergents, kerosene or “oil soaps”.  They may damage the wood and the finish and may leave a discolouring residue. The specialised cleaner specifically recommended for your floor coating is ideal.

- Don’t drag anything across the timber flooring, this can result in scratches, gouges & dents in the floor.

EQUIPMENT, MATERIALS AND PERIODICITY

Vacuums and dry mops are both dry cleaning methods. Dry mops are not dipped in a bucket of water – water is sprayed on the floor or on to the mop head. We recommend a mop with a flat head and removable light cover/s. These mops are available from Just Hardwood Floors.

When wet mopping, a good quality mop which rolls/squeezes out almost all of the water is important. If using a wet mop a higher concentration of solution (2-4 times stronger) is required to stop water spots being left behind as the floor dries (cleaning solutions have a drying agent which stops water spots).

The suggested frequency for cleaning a polyurethaned timber floor is simple… as often as you would like. Vacuum or sweep the floor regularly to remove loose dirt, dust and gritty particles prior to mopping. OR, for light cleaning use a dry mop with an anti-static cover for picking up dust. If you are using the correct cleaning method and solution, even daily cleaning will not damage your floor.  Most cleaning solutions can be diluted for general, heavy and spot cleaning.   The regularity of floor cleaning is often tied to the traffic level over the floor and the frequency of vacuuming.

POLYURETHANED SOLID TIMBER FLOORING

Duracoat Floor Cleaner is the recommended product for cleaning solvent based polyurethaned floors.  The solution for normal cleaning with a wet mop is 50ml per 5L water.  For kitchen work, heavy build-up areas, dry mopping and spot cleaning, a solution of 1 part Duracoat to 5 parts water in a spray bottle. For hard to clean areas such as in front of the oven, Duracoat concentrate can be sprayed on in a fine mist and wiped off with a dry or damp cloth.  Duracoat is an extremely effective household cleaner useful for cleaning all resilient surfaces.  Duracoat is available from Just Hardwood Floors for $15.00 per 1L bottle or $45.00 per 5 L bottle.

Floor Cleaning Products

Duracoat Floor Cleaner is the recommended product for cleaning moisture cured polyurethaned floors.

Duracoat can be used as full strength for hard to clean areas or heavily diluted for lighter cleaning. Unlike

most other cleaning solutions, it will not leave a build-up or residue on the floor and it contains a drying agent to

prevent water marks being left after cleaning.

Duracoat is generally used diluted 1:5 parts water as a spot cleaner or in a “spray n wipe bottle”.

1 Litre Bottle$15.00 incl.

5 Litre Bottle$45.00 incl.

Sabco Mop with Wet and Dry Covers$40.00 incl.

Replacement Wet Covers$15.00 incl.

Maintenance Products  (prices as at 1 March 2018)

The following products, available from our showroom, are recommended for use with solid timber, timber veneer and laminate flooring.  Visit us or phone through your order and we can courier your order (delivery charge applies).

Feltac Felt Pads

We highly recommend Feltac self-adhesive pads to prevent your furniture damaging your timber floor. Feltac pads are a heavy duty, dense, self-adhesive felt pad. Just peel off the backing and stick them to the bottom of your furniture. They come in a wide range of sizes that will suit anything you have in your home.

Felt pads should be placed on movable items (eg chairs) to prevent scratches and on non-movable items (eg tables and cabinets) to prevent dents. The felts will not compress with age and the self adhesive layer bonds firmly so they offer quality, durable protection.

Protects floors and walls		Adds feeling of softness

Superior quality			Makes furniture easier to move

Ultra-strength adhesive		Highly durable

19mm Round Discs Beige (20 Pieces)		17048		$6.50 incl.

25mm Round Discs Beige (16 Pieces) 		17042		$6.50 incl.

38mm  Round Discs Beige (6 Pieces) 		17043		$6.95 incl.

114X152mm Two Piece Rectangle Beige		17041		$10.95 incl.

Various Multipack Beige (33 Pieces)		23054		$12.50 incl.